# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA1201 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL3- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | MAHALAKSHMI S | Reg.No.: | 192524372 |
| Department: | B. Tech AI&DS | Date: | 24/08/26 |

### ANNEXURE A COMPARATIVE ANALYSIS

- Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

#### Answer

Cache memory is organized into multiple levels to reduce the average time required by the CPU to access frequently used data. L1, L2, and L3 differ mainly in size, latency, bandwidth, and proximity to the processor.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Closest to CPU core | Near/within CPU co | Usually shared among cores |
| Typical Size | 16 KB–128 KB per core | 256 KB–2 MB per c | Several MB to tens of MB |
| Latency | ~1–4 cycles | ~5–15 cycles | ~20–50+ cycles |
| Bandwidth | Highest | High | Lower than L1/L2 |
| Access Speed | Fastest | Very fast | Slower than L1/L2 |
| Main Purpos | Immediate data/instruction a | Secondary fast stora | Shared data and reducing accesses |

L1 cache provides the lowest latency and highest effective bandwidth because it is located closest to the execution units. It is usually divided into instruction and data caches. Its small capacity allows very fast access.

L2 cache is larger than L1 but has higher latency. It stores data that cannot fit in L1 and acts as an intermediate buffer before accessing lower-level memory.

L3 cache is significantly larger and is commonly shared between processor cores. Although it has higher latency and lower bandwidth than L1 and L2, it reduces expensive accesses to main memory.

#### Impact on processor performance

The hierarchy improves performance because most frequently accessed data can be found in the faster cache levels. A typical access sequence is:

CPU → L1 → L2 → L3 → DRAM

A high L1/L2 hit rate reduces the average memory access time. L3 is especially important in multicore processors because shared data can be accessed without immediately going to DRAM.

For example, if L1 access takes about 2 cycles while DRAM access takes roughly 100+ cycles, avoiding a DRAM access can save a large number of CPU cycles.

Critical analysis: Increasing cache size does not automatically increase performance. Larger caches generally have greater latency and hardware cost. Therefore, modern processors use a hierarchy that balances speed, capacity, bandwidth, and cost. This directly supports the CO4 objective of implementing hierarchical memory systems based on latency, bandwidth, and throughput.

- Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

#### Answer

SRAM and DRAM are both volatile memory technologies, but their different internal structures make them suitable for different levels of the memory hierarchy.

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Storage Cell | Flip-flop, typically 6 transistors | Capacitor + transistor |
| Latency | Very low | Higher |
| Bandwidth | Very high | High, but generally lower effective latency |
| Throughput | Very high for low-latency accesses | High for large sequential transfers |
| Refresh | Not required | Required periodically |
| Density | Lower | Higher |
| Cost per bit | Higher | Lower |
| Typical Use | CPU caches | Main memory |

#### SRAM in cache design

SRAM is preferred for L1, L2 and many L3 caches because it can provide fast access without refresh operations. Its low latency allows the processor to retrieve frequently used instructions and data quickly. However, SRAM requires more transistors per bit and therefore occupies more chip area. This makes large SRAM caches expensive.

#### DRAM in main memory

DRAM stores each bit using a transistor and capacitor. Because the capacitor gradually loses its charge, DRAM requires periodic refreshing. This increases control complexity and contributes to greater access latency.

Its major advantage is high density and lower cost per bit, making it practical for large main-memory capacities.

#### Performance interpretation

SRAM is optimized for low latency and rapid repeated access, whereas DRAM is optimized for high capacity and economical storage.

For example:

CPU → SRAM Cache → DRAM Main Memory

If requested data is available in SRAM cache, the CPU avoids the considerably slower DRAM access.

Critical analysis: Using SRAM for all main memory would provide excellent latency but would be prohibitively expensive and area-intensive. Conversely, using DRAM for cache would increase processor stalls. Therefore, combining SRAM and DRAM provides an effective balance between latency, bandwidth, capacity, and cost.

- Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

#### Answer

Virtual memory with paging and cache memory both improve system operation by managing different levels of the memory hierarchy, but they solve different problems.

| Feature | Virtual Memory + Paging | Cache Memory |
| --- | --- | --- |
| Purpose | Provides large logical address space | Reduces CPU memory-access late |
| Managed by | OS + MMU | Hardware/cache controller |
| Storage | RAM + secondary storage | SRAM-based cache |
| Access Latenc | Very low for RAM hits, extremely high durin faults | Very low |
| Data Unit | Pages | Cache lines/blocks |
| Miss Penalty | Very high for page fault | Much lower than disk/page-fault p |

| Feature | Virtual Memory + Paging | Cache Memory |
| --- | --- | --- |
| Main Benefit | Memory capacity and process isolation | Speed and CPU performance |

#### Cache mechanism

Cache memory keeps frequently or recently accessed data close to the CPU. When the required data is found, a cache hit occurs. A cache miss requires the processor to access a lower level of the memory hierarchy.

Therefore:

Cache hit → Low latency → Higher CPU throughput

#### Virtual memory and paging

Virtual memory allows programs to use an address space larger than the available physical RAM. The address space is divided into pages, while physical memory is divided into page frames.

When a required page is not present in RAM, a page fault occurs. The operating system must retrieve the page from secondary storage, producing a very large latency compared with cache or DRAM access.

#### Impact on throughput

Cache memory directly improves CPU throughput by reducing the time processors spend waiting for memory.

Virtual memory improves system-level throughput and flexibility by allowing multiple programs to execute while providing isolation and efficient memory allocation. However, excessive page faults can cause thrashing, where the system spends more time transferring pages than executing programs.

Critical analysis: Cache and virtual memory should not be viewed as competing technologies. They operate at different levels of the hierarchy:

CPU → Cache → DRAM → Virtual Memory/Storage

Cache primarily addresses latency, whereas virtual memory primarily addresses capacity, protection, and process management. An efficient system requires both mechanisms.

- Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

#### Answer

NAND Flash and DRAM are both widely used memory technologies, but their characteristics are fundamentally different. DRAM is volatile and optimized for fast working memory, while NAND Flash is non-volatile and optimized for high-density storage.

| Feature | DRAM | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile | Non-volatile |

| Feature | DRAM | NAND Flash |
| --- | --- | --- |
| Latency | Low | Much higher |
| Bandwidth | High | High sequential bandwidth possible |
| Random Access | Very efficient | Relatively slow |
| Write Operation | Fast | Slower and requires program/erase operations |
| Endurance | Very high for normal operation | Limited program/erase cycles |
| Density | Lower | Very high |
| Cost per Bit | Higher | Lower |
| Typical Use | Main memory | SSDs, flash storage |

### DRAM

DRAM provides low-latency random access and is therefore suitable for main memory. The CPU can access individual memory locations efficiently, making DRAM appropriate for active programs and data.

#### NAND Flash

NAND Flash retains data even when power is removed. It provides high storage density and relatively low cost per bit. However, its access characteristics are much slower than DRAM, particularly for small random operations.

NAND Flash is organized around pages and erase blocks. Data writing and erasing involve additional management operations, which contribute to latency.

#### Suitability in hierarchical memory

A typical hierarchy is:

CPU → Cache → DRAM → NAND Flash

DRAM is positioned closer to the CPU because its lower latency makes it suitable for active computation. NAND Flash is positioned farther away because it offers much greater capacity and non-volatility.

Critical analysis: NAND Flash can provide impressive sequential throughput, especially in modern SSDs, but throughput should not be confused with low latency. DRAM remains much more suitable for workloads requiring frequent random accesses.

Thus, DRAM optimizes working-memory performance, while NAND Flash optimizes capacity, persistence, and cost efficiency.

- Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

#### Answer

MRAM (Magnetoresistive RAM) and RRAM (Resistive RAM) are emerging non-volatile memory technologies designed to overcome some limitations of conventional memory systems.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Storage Principl | Magnetic state | Resistance state |
| Volatility | Non-volatile | Non-volatile |
| Access Time | Very low | Potentially very low |
| Read Speed | Fast | Fast |
| Write Speed | Fast | Fast, technology-dependent |
| Throughput | High | High potential |
| Energy Efficienc | Good | Potentially very good |
| Endurance | Generally high | Depends strongly on material/device |
| Scalability | Good | Very high potential |
| Applications | Embedded memory, cac applications | High-density storage and neurom computing |

### MRAM

MRAM stores information using magnetic states rather than electrical charge. Since it is non-volatile, it can retain data without continuous power.

Its major advantage is the combination of fast access, non-volatility, and high endurance. This makes MRAM attractive for embedded systems and memory applications requiring frequent read/write operations.

### RRAM

RRAM stores information by changing the resistance of a material. It can potentially achieve high density because its cell structure can be compact.

RRAM is attractive for future high-density memory systems and specialized computing applications. Its energy consumption can be low, particularly for certain read operations, although write characteristics vary according to the implementation.

#### Performance comparison

MRAM generally offers a strong combination of speed, endurance, and non-volatility, while RRAM has significant potential for high density and energy-efficient operation.

Critical analysis: Neither technology universally replaces SRAM or DRAM. Their suitability depends on workload requirements. MRAM can be attractive where fast access and endurance are important, whereas RRAM can be particularly attractive where density and low-energy operation are priorities.

Therefore, both technologies have potential to improve future hierarchical memory systems by providing a compromise between volatile high-speed memory and slower non-volatile storage.

Code of Conduct:

I MUHITHA K(192524339) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Muhitha k

### MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |

| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| --- | --- | --- | --- | --- | --- |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |