# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 2-DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL3DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | S.MAHALAKSHMI | Reg.No.: | 192524372 |
| Department: | B.TECH AI DS | Date: | 31/07/2026 |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- Identification of Errors / Bugs

  - Inputs:

o +120 = 01111000

o +50 = 00110010

  - Binary Addition:

01111000

+ 00110010

10101010

  - Result = 10101010 = -86 (incorrect)

  - Expected Result = 170

  - Since 170 cannot be represented in 8-bit signed format (-128 to +127), overflow occurs.

### Debugging Approach & Analysis

Check overflow condition. Rule:

  - Adding two positive numbers gives a negative result.

  - OR adding two negative numbers gives a positive result. Here,

Positive + Positive = Negative Hence,

Overflow Detected = YES

### Correctness of Debugged Solution

Overflow detection logic:

Overflow =

(A7 == B7) AND (Result7 != A7)

Where A7, B7 and Result7 are the sign bits. For this example,

A7 = 0

B7 = 0

Result7 = 1

Overflow = TRUE

The processor correctly identifies overflow instead of accepting the wrong answer.

### Optimization Techniques Applied

  - Add overflow detection circuitry.

  - Generate overflow flag.

  - Use wider data type (16-bit or 32-bit).

  - Display overflow exception.

### Testing and Documentation

| Test Case | Expected | Result |
| --- | --- | --- |
| 120 + 50 | Overflow | Pass |
| 20 + 30 | 50 | Pass |
| -70 + -80 | Overflow | Pass |
| 40 + -20 | 20 | Pass |

### A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry lookahead adder, and explain how it reduces delay.

- Identification of Errors / Bugs

The ripple carry adder is functionally correct but suffers from high propagation delay because each carry must travel through every full adder.

Example:

Bit0 → Bit1 → Bit2 → ... → Bit7 Each stage waits for the previous carry.

### Debugging Approach & Analysis

Delay increases with the number of bits. Example:

C0

↓ FA0

↓ C1

↓ FA1

↓ C2

↓

... C8

Total delay

Delay = n × Full Adder Delay

### Correctness of Debugged Solution

Replace Ripple Carry Adder with Carry Lookahead Adder. Carry equations:

Pi = Ai XOR Bi Gi = Ai AND Bi

Ci+1 = Gi + PiCi

Carries are generated simultaneously.

### Optimization Techniques Applied

  - Carry Lookahead Adder

  - Parallel carry generation

  - Reduced propagation delay

  - Faster arithmetic operations

### Testing and Documentation

| Adder | Delay | Performance |
| --- | --- | --- |
| Ripple Carry | High | Slow |
| Carry Lookahead | Low | Fast |

Performance improved significantly.

### A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bitpair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- Identification of Errors / Bugs

Incorrect multiplication occurs because of:

  - Wrong bit-pair checking

  - Incorrect arithmetic right shift

  - Improper sign extension

Negative operands produce incorrect results.

### Debugging Approach & Analysis

Verify each iteration.

Check:

Q0 Q-1

Rules

01 → Add M

- → Subtract M 00 → No operation

- → No operation After operation, Perform

Arithmetic Right Shift

Ensure sign bit remains unchanged.

### Correctness of Debugged Solution

Corrections:

✔ Correct bit-pair detection

✔ Preserve sign bit

✔ Correct arithmetic shift

✔ Proper sign extension

Now negative multiplication produces correct answer.

### Optimization Techniques Applied

  - Correct control logic

  - Automatic sign extension

  - Validate each iteration

  - Error checking after every shift

### Testing and Documentation

| Operands | Expected | Result |
| --- | --- | --- |
| 5 × 3 | 15 | Pass |
| -5 × 3 | -15 | Pass |
| -5 × -3 | 15 | Pass |
| 7 × -4 | -28 | Pass |

### An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- Identification of Errors / Bugs

The algorithm restores the previous remainder whenever subtraction becomes negative. This causes:

  - Extra additions

  - Increased execution time

### Debugging Approach & Analysis

Workflow:

Subtract

↓ Negative?

↓ Yes

↓ Restore

↓ Repeat

Repeated restoration is inefficient.

### Correctness of Debugged Solution

Replace Restoring Division with Non-Restoring Division. Instead of restoring:

Negative

↓

Next iteration performs addition No immediate restoration.

Produces the same quotient with fewer operations.

### Optimization Techniques Applied

  - Non-Restoring Division

  - Eliminate unnecessary restoration

  - Reduce arithmetic operations

  - Improve execution speed

### Testing and Documentation

| Method | Operations | Speed |
| --- | --- | --- |
| Restoring | High | Slow |
| Non-Restoring | Low | Fast |

Execution time reduced.

### A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

  - Identification of Errors / Bugs

Example:

1 × 10^30

+ 1

The small value disappears after exponent alignment. Causes:

- Loss of precision

- Rounding error

- Normalization issues

### Debugging Approach & Analysis

Floating-point addition process:

Align Exponents

↓

Shift Mantissa

↓

Add Mantissas

↓ Normalize

↓ Round

Large exponent difference causes the smaller mantissa to be shifted out.

### Correctness of Debugged Solution

Improve accuracy by:

  - IEEE 754 Double Precision (64-bit)

  - Kahan Summation Algorithm

  - Higher precision arithmetic

Results become significantly more accurate.

### Optimization Techniques Applied

  - Double precision

  - Kahan summation

  - Pairwise summation

  - Improved rounding methods

### Testing and Documentation

| Precision | Result | Accuracy |
| --- | --- | --- |
| `Single Precision | Precision Loss | Low |
| Double Precision | Accurate | High |

The optimized approach minimizes rounding errors and improves numerical stability.

Code of Conduct:

I D.G.RAGHAVI(192521173) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: DG.RAGHAVI

### MARKS ALLOCATION

|  | Identification of Errors / Bugs(20%) | Debugging Approach & Analysis(20%) | Correctnes s of Debugged Solution(2 5%) | Optimization Techniques Applied(20%) | Testing and Documentation(15 %) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |